# Robot

# Event

One way distance: 273m

Total distance: 546m

Single run time: 1h

Time for maintenance: 30min

Corridor narrows to 125cm near the ramp.

You can set GPS perimeter 1h - 15m before start

Velocity:

Pure minimum avg_vel = 0.152m/s

15min to spare: avg_vel = 0.202m/s  (Minimal)

30min to spare: avg_vel = 0.303m/s  (Good)

45min to spare: avg_vel = 0.627m/s  (Speed)

Search area:

~10x10m

Cosmap_2d:

Not rotated:

180x100m  18000m2

Cell size 0.025m: 7200x4000 -> 110MiB (real 28.8MiB)

Cell size 0.050m: 3600x2000 -> 27.5MiB (real 7.0MiB)

Rotated:

170x30m   5100m2

Cell size 0.025m: 6800x1200 -> 31.1MiB

Cell size 0.050m: 3400x600   -> 7.8MiB

Actually used map (padded about 7m in all sides to account for uncertainty):

124x175m  21700m2

Cell size 0.1m: 1240x1750 -> 2.2MiB

# New PC setup

(not complete; cd directories are example and might be different; not all steps are necessary)

- Install Arduino IDE

  - sudo snap install arduino  # For newest version (on apt it’s old 2:xx)

  - https://ubuntu.com/tutorials/install-the-arduino-ide#2-installing-via-a-tarball

  - Setup STM32 stuff

    - https://maker.pro/arduino/tutorial/how-to-program-the-stm32-blue-pill-with-arduino-ide

- Install vscode

  - Extensions: Arduino, C/C++, Git Graph, Python, Todo Tree

  - ROS setup: https://github.com/RoboGnome/VS_Code_ROS#1-vs-code-extensions

- Install PyCharm Community

  - From GUI Ubuntu Software

  - Increase max max inode wathces https://unix.stackexchange.com/questions/13751/kernel-inotify-watch-limit-reached

- Reduce Ubuntu padding by creating file at ~/.config/gtk-3.0/gt:

/* Reducing padding */ .default-decoration { min-height: 0px; padding: 0px; } .default-decoration .titlebutton { min-height: 0px; padding: 0px; } .header-bar.default-decoration { padding-top: 0px; padding-bottom: 0px; } .header-bar.default-decoration .button.titlebutton { padding-top: 0px; padding-bottom: 0px; } /* No line below the title bar */ .ssd .titlebar { border-width: 0px; box-shadow: none; }

- Install ROS Melodic

  - http://wiki.ros.org/melodic/Installation/Ubuntu

  - Update project dependencies

# Check missing dependencies rosdep check --from-paths ~/Documents/SamanaAutonomousRobot/ROS/samana_ws/src/ --ignore-src # Check all dependencies (only installed?) rospack depends samana # Simulate what dependencies will be installed rosdep install --simulate --from-paths ~/Documents/SamanaAutonomousRobot/ROS/samana_ws/src/ --ignore-src # Install missing dependencies rosdep install --from-paths ~/Documents/SamanaAutonomousRobot/ROS/samana_ws/src/ --ignore-src # Or simplest form rosdep install samana --ignore-src

  - Manual dependencies

sudo apt install nvidia-container-toolkit sudo apt install ros-melodic-sbpl-lattice-planner sudo apt install feh

- Give usb permissions when connected all Arduinos

sudo chmod 0777 /dev/ttyUSB*

- Install Roboware Studio

  - Files here: https://github.com/TonyRobotics/RoboWare

  - On ERROR: bash: catkin_make: command not found

    - Click this icon

    - ctrl + f and find “catkin_make” in tasks.json

    - Add source ros to before catkin_make. Also add to release task

    - It should look like:

"taskName": "Debug", "args": [ "source /opt/ros/melodic/setup.bash; catkin_make -C ${workspaceRoot} -DCMAKE_BUILD_TYPE=Debug" ],

- Make symbolic links of Arduino libraries to actual path where Arduino libraries are read (might need editing make_links to point to right location)

  - cd /home/combinacijus/Documents/SamanaAutonomousRobot/Arduino/

  - sh make_links.sh

- Copy udev rules from Other Files

cd /home/combinacijus/Documents/SamanaAutonomousRobot/OtherFiles/UbuntuCofigs/UdevRules sudo cp 49-stlinkv2* 99-usb-serial.rules /etc/udev/rules.d/ sudo udevadm control --reload

- Install Docker (and run here)

  - https://docs.docker.com/engine/install/ubuntu/

# Add permissions to edit Docker config file sudo chown "$USER":"$USER" /home/"$USER"/.docker -R sudo chmod g+rwx "/home/$USER/.docker" -R # Add the package repositories # On Error when running docker run below commands (should be found online): # docker: Error response from daemon: could not select device driver "" with # capabilities: [[gpu]]. distribution=$(. /etc/os-release;echo $ID$VERSION_ID) curl -s -L https://nvidia.github.io/nvidia-docker/gpgkey ¦ sudo apt-key add - curl -s -L https://nvidia.github.io/nvidia-docker/$distribution/nvidia-docker.list sudo tee /etc/apt/sources.list.d/nvidia-docker.list sudo apt-get update && sudo apt-get install -y nvidia-container-toolkit sudo systemctl restart docker # Nvidia driver dependency sudo apt install nvidia-driver-440 sudo apt install nvidia-container-toolkit # Download image for CUDA docker pull combinacijus/imageai:compute3.0

- Git and git LFS

  - https://git-lfs.github.com/

# Git sudo apt-get install git # git LFS curl -s https://packagecloud.io/install/repositories/github/git-lfs/script.deb.sh ¦ sudo bash sudo apt install git-lfs

- Build catkin workspace setup

  - sudo apt-get install make

  - On Error: Makefile:140: recipe for target 'all' failed

On Error failed catkin_make: make[2]: *** No rule to make target '/usr/lib/x86_64-linux-gnu/libGL.so', needed by '/home/combinacijus/Documents/test_ws/devel/lib/librviz_imu_plugin.so'.  Stop.

https://github.com/RobotLocomotion/drake/issues/2087#issue-148166827

# Check does this link point to existing file or that file to existing file etc readlink /usr/lib/x86_64-linux-gnu/libGL.so # Remove this link if not sudo rm /usr/lib/x86_64-linux-gnu/libGL.so # Find where libGL.so.1 is located sudo find / -name "libGL.so*" # Relink it (might need changing directories) sudo ln -s /usr/lib/x86_64-linux-gnu/libGL.so.1 /usr/lib/x86_64-linux-gnu/libGL.so

- git clone [Samana repository] and build

mkdir /home/combinacijus/Documents/SamanaAutonomousRobot cd /home/combinacijus/Documents/SamanaAutonomousRobot git clone https://github.com/Combinacijus/Samana-Autonomous-Robot.git cd ./ROS/samana_ws catkin_make

- ~/.bashrc

# ROS source "/opt/ros/melodic/setup.bash" source "/home/combinacijus/Documents/SamanaAutonomousRobot/ROS/samana_ws/devel/setup.bash" source "/home/combinacijus/Documents/ROS/learning_tf_ws/devel/setup.bash" export TURTLEBOT3_MODEL=waffle_pi stty -ixon export PATH=/usr/local/cuda-10.0/bin${PATH:+:${PATH}} export LD_LIBRARY_PATH=/usr/local/cuda-10.0/lib64${LD_LIBRARY_PATH:+:${LD_LIBRARY_PATH}}

- Free up space on /home partition by symlinking to / partition cache and other directories

# Directory setup sudo mkdir /usr/home-partition sudo chown combinacijus /usr/home-partition/ sudo chmod 775 /usr/home-partition/ cd /usr/home-partition # Free up space sudo baobab # To find disk usage # Create directory for VSCode workspace files sudo mkdir /usr/home-partition/.config/Code/User/workspaceStorage/ # Via gui copy files to /usr/home-partition/<filepath-after-home> # Symlinks. .cache is regenerating so run fast (after redeleting) sudo ln -s /usr/home-partition/.config/Code/User/workspaceStorage/ ~/.config/Code/User/ sudo ln -s /usr/home-partition/.cache/ /home/combinacijus/

# Git

- To push single branch to remote: (origin = Samana-...

git checkout feature_x git push origin -u feature_xg

  - use -f if you need to overwrite backup branch

- Undo last commit:

git reset --soft HEAD~1

- To move commits from backup_branch to other branch:

  - Go to new branch

  - Cherry pick commits you want

  - Delete backup_branch

  - Create backup_branch

Merge many commits:

git reset HEAD@{n} # n found from git reflog git commit

delete branch local and remote:

# delete branch locally git branch -d localBranchName # delete branch remotely git push origin --delete remoteBranchName git push origin :remoteBranchName

git lfs:

git lfs track "**/*.lfs" # This generates .gitattributes Or edit it git add .gitattributes git add file_to_be_in_lfs git lfs ls-files # Should list all LFS file already pushed and newly staged git lfs status # Should show files with LFS: tag

### git reset

Good for tidying up commits to single commit or to many commits

A - B - C - D <- HEAD

git reset vs git rm --cached

git rm --cached, puts specified files into untracked, and stages their removal for the next commit.

git reset --mixed, simply moves the file into 'unstaged', but keeps its old version in repo (opposite of git add).

# GPS

- Find IP in terminal write ip route and find last ip scope link src 192.168.0.102 metric append :11311/

- Connect via ROS Sensors Driver on Android with same IP eg. http://192.168.0.102:11311

- No error should be shown

# In ROS run these commands export ROS_MASTER_URI=http://192.168.0.102:11311/ # Not necessary? roscore rosrun topic_tools trsform /android/fix /android/fix/fixed sensor_msgs/NavSatFix 'sensor_msgs.msg.NavSatFix(header=std_msgs.msg.Header(seq=m.header.seq,stamp=m.header.stamp,frame_id="gps"), status=m.status, latitude=m.latitude, longitude=m.longitude, altitude=m.altitude, position_covariance=m.position_covariance, position_covariance_type=m.position_covariance_type)' --import sensor_msgs std_msgs

- Lowest covariance on gps module at night: 15500.25 (very bad)

- Lowest covariance on a smartphone at night: 18.38 (fair?)

# General programming

- IMPORTANT: sync clocks with ROS clock

- F() Macro on Arduino printing. To save SRAM

- nh.getHardware()->setBaud(1000000);

- rosrun rosserial_arduino serial_node.py _baud:=115200 _port:=/dev/ttyUSB0

- delay(1);

- Be careful with rosserial because it can desync if data is sent too fast

- 92% memory usage causes instability

- Lost sync with device, restarting…

  - Lower baud rate

- To build ROS library for Arduino

- build samana_msgs

source devel/setup.bash rm -r ~/Documents/SamanaAutonomousRobot/Arduino/libraries/ros_lib rosrun rosserial_arduino make_libraries.py ~/Documents/SamanaAutonomousRobot/Arduino/libraries/

- If can’t include headers first include ros.h

- wiki.ros.org/rosserial_arduino/Tutorials/Arduino%20IDE%20Setup

- Further if adding customized ros_lib for a node:

  - In Samana project main folder /Arduino modify make_links.sh to create a symbolic link of ros_lib_newnode to a Arduino/libraries folder on computer (place where IDE reads libraries) e.g.

ln -sv /home/combinacijus/Documents/SamanaAutonomousRobot/Arduino/libraries/ros_lib_odom/ /home/combinacijus/Documents/Arduino/libraries/

- If ROS workspace change or adding new

sudo subl .bashrc source "/home/combinacijus/Documents/SamanaAutonomousRobot/ROS/samana_ws/setup.sh"

  - .../build/CMakeCache.txt change all directories to new one

  - Also in RoboWare change settings.json for autocomplete

- If node tried to send before init “Tried to publish before configured”:

  - Build project (catkin_make)

  - Source devel/setup.sh

  - Check rosmsg list if your package messages are loaded

- To accept arduino node to roscore

sudo chmod 666 /dev/ttyUSB* rosrun rosserial_arduino serial_node.py _baud:=10000000 _port:=/dev/ttyUSB0 _recal:=false

- UDEV rule for separating Aruduino depending on devpath (usb port)

SUBSYSTEM=="tty", ATTRS{idVendor}=="1a86", ATTRS{idProduct}=="7523", SYMLINK+="ttyUSB-arduino%s{/devpath}"

  - This gives names like:  /dev/ttyUSB-arduino2.1  /dev/ttyUSB-arduino2.3.3

# General electronics:

- IMPORTANT: Capacitor on input to eliminate power loses

# IMU BNO055

- SCL: A5; SDA: A4, 5V, GND

- Human ride: max angular velocity 300deg/s

- Human ride: max acceleration 3m/s^2

- BNO055 loses calibration when space between hoverboard is <20cm (30cm from any electronic would be nice)

- On system calibration loss. 360 + 180 - 180 is advised to recalibrate IMU

- IMPORTANT: On magnetometer calibration degradation one wheelie to different headings might calibrate it

- IMPORTANT: After loosing power IMU won’t recover. Needs extra code. Problem with Wire.h library in twi.cpp file, issue: https://github.com/panStamp/arduino_avr/issues/1. Maybe needs extra Arduino to reset via pin.

- PROBLEM: Heading is not reliable

- Baud rate could be lowered to 57600 but it will reduce update speed from 100hz to 73hz

# Distance sensors

- Max distance 4-6m at home

- Need spacer in front so minimum distance wouldn’t be 0. Else it would cause bugs

- SRF04 will only listen to echo if it was triggered

- Ultrasonic sensors on other robots might trigger own sensors therefore giving shorter distance to the obstacle

- IMPORTANT: deal with bump sensors when angled

- avrdude: stk500_recv(): programmer is not responding. But will upload via Arduino IDE

- IMPORTANT: ROS: check each sensor max distance. If it’s too big map won’t be accurate it will have wall at the end of sonar cone

# Hoverboard

- IMPORTANT: There is inactivity timeout on hoverboard firmware. It probably should be disabled for competition

- Main battery voltage screen shows 0.4V lower voltage

- Battery:

- 3.6V Low voltage no beep

  - 3.5V Almost empty battery; beeps

  - 3.37V Turns offs when almost no speed

  - Charging speed. no_load=35.1V to no_load=38.6V in 1hr; to 41.6V 1h45m

# Arm

- IMPORTANT: if motors disables on their own it might be because overcurrent protection

- If motors don’t work all manual switches might be on

Times with no extra weight and grabber on lifter down position:

|  | Dry thread | Lubed thread | Lubed long ago |
| --- | --- | --- | --- |
| Lifter up | 18s | 18s |  |
| Lifter down | 13s | 13s |  |
| Grabber open | 11s |  |  |
| Grabber close | 11s |  |  |

# Object recognition

- Pretrained YoloV3: https://github.com/OlafenwaMoses/ImageAI/releases/tag/essential-v4

- You can keep the images at their original size. The training network will automatically re-scale the image to sizes between 288 to 488 as it runs through many experiments.

- Too large images will exhaust GPU memory

- On CPU 0.34 - 0.42 FPS on 100%; on GPU

- GPU worked on nvidia-smi Driver Version: 390.132  440

- On ERROR: socket.error: [Errno 98] Address already in use. Run   lsof -i :5005   and check listening processes and sudo kill -9 <process_id>

- To connect Android phone as IP camera create hotspot on computer and connect to it via phone's wifi

  - To create hotspot go to: Wifi Settings -> Menu button at the top right corner -> Turn on Wi-Fi Hotspot

### Google Colab local runtime

jupyter notebook --NotebookApp.allow_origin='https://colab.research.google.com' --port=8888 --NotebookApp.port_retries=0 --allow-root

### Docker

sudo docker run -it --rm --gpus all --net host -v /home/combinacijus/Documents/SamanaAutonomousRobot/Python/GoldBagDetector/:/notebooks/GoldBagDetector -v /home/combinacijus/Desktop/ros-logs/:/home/combinacijus/Desktop/ros-logs/ -w /notebooks/GoldBagDetector --name imageai combinacijus/imageai:compute3.0 sudo docker run -it --rm --gpus all --net host --device /dev/video2 -v /home/combinacijus/Documents/SamanaAutonomousRobot/Python/GoldBagDetector/:/notebooks/GoldBagDetector -w /notebooks/GoldBagDetector --name imageai combinacijus/imageai:compute3.0 cd /notebooks/GoldBagDetector/ python GoldBagDetector.py # Run ROS script (TCP/IP server side) # Set laptop to performance mode feh -R 0.7 /home/combinacijus/Documents/SamanaAutonomousRobot/Python/GoldBagDetector/image.jpg # For image debugging sudo docker run -it --gpus all -v ~/Documents/SamanaAutonomousRobot/Python/GoldBagDetector/:/notebooks/GoldBagDetector imageai sudo docker run --gpus all -it --rm tensorflow/tensorflow:1.12.0-gpu /bin/bash sudo docker run --gpus all -it -v /host_path:/container_path tensorflow/tensorflow:1.12.0-gpu /bin/bash sudo docker images sudo docker ps -a # Show running containers sudo docker start -ai CONTAINER sudo docker commit CONTAINER NEW_IMAGE_NAME # eg. sudo docker commit focused_taussig combinacijus/imageai:compute3.0 sudo docker rmi IMAGE sudo docker stop CONTAINER sudo docker login # Works sudo docker login docker.io sudo docker tag ID combinacijus/imageai sudo docker push combinacijus/imageai

### To update Docker image

sudo docker login sudo docker run -it --name imageai combinacijus/imageai:compute3.0 # In Dokcer run commands to install packages needed # Press Ctrl+D to exit container sudo docker ps -a # Check name or ID # (template but check names before running) sudo docker commit CONTAINER NEW_IMAGE_NAME sudo docker commit imageai combinacijus/imageai:compute3.0 sudo docker push combinacijus/imageai:compute3.0

ubuntu

- On Error: docker: Error response from daemon: OCI runtime create failed: container_linux.go:349: starting container process caused "process_linux.go:449: container init caused \"process_linux.go:432: running prestart hook 0 caused \\\"error running hook: exit status 1, stdout: , stderr: nvidia-container-cli: initialization error: driver error: failed to process request\\\\n\\\"\"": unknown.

# Install Nvidia driver and reboot computer sudo apt install nvidia-440

### Training new model

- Take photos with the ‘Open Camera’ app. Around 200 for an object.

- Set resolution to 640x480.

- Photos can be taken with bluetooth headphones volume buttons.

- Folder structure:

  - dataset1

    - train

      - images

        - IMG001.jpg

        - IMG002.jpg

      - annotations

        - IMG001.xml

        - IMG002.xml

    - validation

      - images

        - IMG003.jpg

      - annotations

        - IMG003.xml

- Separate images to validation set now or do it automatically randomly later.

- Rename all images to IMGXTxxx.jpg eg. IMG2T001,jpg IMG2V023.jpg. X - dataset number, T - type, either T for train or V for validation, x - images number or XTxxx - any images name could be default name.

- Annotate images with labelImg. Class name ‘goldbag’. View-> Single Class Mode. View -> Auto Save Mode. https://medium.com/deepquestai/object-detection-training-preparing-your-custom-dataset-6248679f0d1d

- Edit ‘‘seperate-validation-images.py’ by reading ‘NOTE:’ comments. Change directories to right ones.

- Run ‘seperate-validation-images.py’ script to separate around 20% of images and annotations to validation set.

- Upload data set to Google Drive in GoldBagDetector directory

- Go to jupyter notebook: https://colab.research.google.com/drive/17DeoyW0hmTHndsOTblY4hXfw0GV6wFmA#scrollTo=F_8qYPEJt3eg

- Change directories noted by ‘NOTE:’ comments

- Run javascript command to imitate activity so that max limit would be 12hr instead of 90min

https://stackoverflow.com/questions/57113226/how-to-prevent-google-colab-from-disconnecting

function ConnectButton(){

console.log("Connect pushed");

document.querySelector("#top-toolbar > colab-connect-button").shadowRoot.querySelector("#connect").click()

}

setInterval(ConnectButton,60000);

- Run scripts in order to train the model

- One epoch takes 12min to train. train: 391, validation: 97, batch_size=4, GPU

One epoch takes 15min to train. train: 467, validation: 126, batch_size=4, GPU

- Training is done in around 10-16 epochs (over 2h)

# ROS

Python nodes might get stuck running after shutting down launch file and would eat a lot of resources in that case:

kill -9 <PID>

To visualise range data (launch file):

- Make URDF model

- Find and Load URDF model to param server:

<arg name="model" default="$(find urdf_tutorial)/urdf/01-myfirst.urdf"/> <param name="robot_description" command="$(find xacro)/xacro.py $(arg model)" />

- For loading rviz config:

m<arg name="rvizconfig" default="$(find urdf_tutorial)/rviz/urdf.rviz" />

- Publish joint states (all rotation of the robot etc):

<node name="joint_state_publisher" pkg="joint_state_publisher" type="joint_state_publisher" />

- Publish all TFs of the robot:

<node name="robot_state_publisher" pkg="robot_state_publisher" type="state_publisher" />

- Start rviz:

<node name="rviz" pkg="rviz" type="rviz" args="-d $(arg rvizconfig)" required="true" />

- Full example launch file:

<launch> <arg name="model" default="$(find urdf_tutorial)/urdf/01-myfirst.urdf"/> <arg name="gui" default="true" /> <arg name="rvizconfig" default="$(find urdf_tutorial)/rviz/urdf.rviz" /> <param name="robot_description" command="$(find xacro)/xacro.py $(arg model)" /> <param name="use_gui" value="$(arg gui)"/> <node name="joint_state_publisher" pkg="joint_state_publisher" type="joint_state_publisher" /> <node name="robot_state_publisher" pkg="robot_state_publisher" type="state_publisher" /> <node name="rviz" pkg="rviz" type="rviz" args="-d $(arg rvizconfig)" required="true" /> </launch>

- To check xacro validity in terminal (cd to urdf file directory):

check_urdf <(xacro samana.xml)

- XML might be invalid because comment ending with triple minus --->

- On Error failed catkin_make: make[2]: *** No rule to make target '/usr/lib/x86_64-linux-gnu/libGL.so', needed by '/home/combinacijus/Documents/test_ws/devel/lib/librviz_imu_plugin.so'.  Stop.

https://github.com/RobotLocomotion/drake/issues/2087#issue-148166827

# Check does this link point to existing file or that file to existing file etc readlink /usr/lib/x86_64-linux-gnu/libGL.so # Remove this link if not sudo rm /usr/lib/x86_64-linux-gnu/libGL.so # Find where libGL.so.1 is located sudo find / -name "libGL.so*" # Relink it (might need changing directories) sudo ln -s /usr/lib/x86_64-linux-gnu/libGL.so.1 /usr/lib/x86_64-linux-gnu/libGL.so

- To filter and extract bag data to the file. bag to file csv

# cd to directory rosbag filter bags/Full_room_drive.bag bags/full_room_sonar_1.bag "topic == '/bump' or topic == '/sonar' or topic == '/tf' or topic == '/tf_static'" rostopic echo -b ./bags/Full_room_drive.bag -p /sonar > sonar_data.csv

Localization ekf navsat nav_sat:

# Gazebo:

- Turtlebot3 tutorial: https://emanual.robotis.com/docs/en/platform/turtlebot3/simulation/#gazebo-3d

# STM32 in Hoverboard

- To stm32 flash:

SWDIO darker gray wire, GND blue, SWCLK lighter gray

Don’t connect 3.3V!!!

Flashing sequence:

- Test jumper wires on stm32 bluepill board

- Might need to unscrew hoverboard cover

- Connect 3 wires: GND SWDIO SWCLK to board and st-link V2 (NO 3.3V)

- Hold hoverboard power button for all steps forward (or it will turn off)

- Connect st-link to pc. By probing it should show high-density device

- Flash firmware with command below

watch -n 0.2 st-info --probe # Loops command every 0.2 sec # cd to project folder cd ~/Documents/SamanaAutonomousRobot/Other\ Files/hoverboard-firmware-niklas/ # Rebuild code make clean; make # Flash to stm st-flash --reset write build/hover.bin 0x8000000

- STM32 config file if coreid is unexpected

/usr/share/opencd/scripts/target/stm32f1x.cfg

- Ubuntu:

  - Write code in Arduino. Sketch -> Export compiled Binary

  - cd to binary

st-flash --reset write <binary_name>.bin 0x8000000

- /opt/STM32CubeProgrammer/bin/STM32CubeProgrammer

- To upload bootloder (On Window easy):

  - Download bootloader generic_boot20_pc13.bin (if led pin13):

    - https://github.com/rogerclarkmelbourne/STM32duino-bootloader/tree/master/binaries

  - Flash it in any way to the board (did with stlinkv2 and windows st-link util). Worked with both boot pins on 0

  - If DFU device not detected download and install driver (.bat both)

    - https://codeload.github.com/rogerclarkmelbourne/Arduino_STM32/zip/master

# Odometry

- Max forward speed 9 rps (rotations per second)

- Max one wheel speed turning 19 rps

- To measure covariances UMBMark test

# Hardware

- IMPORTANT: USB hub requires external power supply. Connecting the second laptop USB with double USB A works. Or connect power bank

- Raising the arm with 1kg object uses under 1000mA If pushed could raise around 2kg in the middle of the gripper. To fix it steal plate might be enough

- Weak point of lifter is it’s rack gear because under >1kg of weight it bends a lot

- Lowering without object uses <400mA

# Mapping

- range_sensor_layer for making costmap_2d with sonar sensors

- Some commands

# To clear costmaps (might not show instantly) rosservice call /move_base/clear_costmaps "{}" # To save local costmap rosrun map_server map_saver --occ 100 --free 10 -f map_tmp map:=/move_base/local_costmap/costmap

- symbol lookup error: /opt/ros/melodic/lib//librange_sensor_layer.so

  - Probably package is not installed correctly (from source or from apt) or not built or setup.bash not sourced

# Trivia

- Hoverboard automatically turns off after around 3h +-200min of inactivity (no movement command received)

- Size/Dimesions LxWxH 96x68x90cm

- Size/Dimesions 2D WxH 63x58cm

- Hoverboard max speeds:

  - No load (wheel of the ground):

    - Velocity -4.4m/s - 4.6m/s

    - Angular speed -18rad/s - 15rad/s (-5.7rot/s - 4.8 rot/s)

  - With laptop PID tuned:

    - Velocity 3m/s (tested with velocity pid on, should go a bit faster because pwm was at 80%)

    - Angular speed 7rad/s (only 40% pwm)

- Hoverboard min speeds:

  - With laptop (pid tuned):

    - Velocity 0.02m/s (or maybe even lower with precise input?)

    - Angular speed 0.02rad/s (or maybe lower?)

- Pid brake distance:

  - 0.5-0.7m/s - 14cm

  - 0.3m/s - 9cm

- PID acceleration:

  - 6-8 m/s^2

- Amp draw from the battery at 38V:

  - Minimal: 41 mA | 1.6 W

  - With hoverboard on: 174 mA | 6.6 W

  - With hov and arduinos on: 235 mA | 8.9 W

  - Grabber motor free: 40-120 mA | 1.5-4.7 W (more when mechanism get stuck a bit)

  - Lifter motor

    - Free running: 50 mA | 1.9 W

    - Stall: ~500 mA | 19 W

  - Computer: 1215 mA | 46.2 W

  - Total for all electronics and computation: ~1450 mA | 55.1 W

  - Driving

    - Flat floor walking pace: ~150 mA | 5.7 W

    - High load: >4000 mA | >152.0 W and easily more

- Max speed and rotation: 3.7 m/s and 7 rad/s

# Tips

- Use rosrun rosrun rqt_top rqt_top for profiling nodes for CPU and RAM usage

- roswtf for general R debugging

- grep -R “text to find” ./path to find recursively text in all files

# TODO BEFORE COMPETITION

# TODO

- BUILD CATKIN_MAKE WITH RELEASE TAG

- Fully charge hoverboard

- Calibrate IMU heading

- Check laptop power settings: to be energy efficient, no automatic turn off!

- Hang small bag

- Disable manual arm motors control before autonomous mode

- Check if Arduinos are on right ports

- Check transmitter failsafe behaviour

- Change hard coded temperature on distance sensors Arduino for more accurate distance

jupyter notebook # Runs Jupyter notebook in a current directory

# Needed tools and materials

- Screw driver

- Hex tools (for grabber base plates)

- Soldering equipment

- All electronics (voltage regulator, motors, arduinos)

- Wires

- Electrical, masking, double sided tape

- Wrench 8mm?

- Saw for cutting wood

- Plywood and wood material

- Hot glue gun

- Blue foam

- Wood screws

- Scissors, pen

- Two pliers